V1.0

Technical Manual

BW-TM16P series

Network thermal imaging core

Introduction

Feature

BW-TM series non-cold standard motion is a self-developed image processing technology and intelligent algorithm, using infrared lenses with different focal lengths, through professional signal processing circuits and image processing platforms, the target infrared radiation is completely restored to temperature data, and presented as a clear thermal image. We provide customers with customized products and services, as well as application solutions in the fields of power equipment inspection, scientific research, safe production, storage of hazardous materials, hotels, shopping malls and so on.

● Compatible and easy to merge

● Support secondary development for different focal lengths

● Compact size and easy to integrate

● Meet the specific requirements of industrial temperature measurement

● High sensitivity, NETDS50mK

● Accurate temperature measurement, support multi-gear temperature measurement

Characteristic

Performance Indicators

| Network mobile |  |
| --- | --- |
| Product Model | BW-TM16P |
| Detector Type | Uncooled Vanadium Oxide Infrared Focal Plane Detectors |
| Resolution | 640×512 |
| Pixel Size | 12μm |
| Response Band | 8 ~14μm |
| Detector Noise Equivalent Temperature Difference | ≦40mK@25℃ |
| Images and videos |  |
| Contrast and brightness | Automatic manual |
| Palette | white-hot, black-hot, iron-red, red-hot, rainbow and other |
| Image processing | unevenness correction |
|  | Intelligent gain control |
|  | Detailed hybrid enhancement |
|  | 3D noise reduction |
| Video Type | H264, Mainstream: 720 x 576 @ 25fps |
| Temperature Measurement Performance |  |
| Temperature measurement range | -20℃ ~ +150℃ , 0℃ ~ +550℃ |
| Temperature measurement accuracy | ±2°C or ±2% (from 20°C to 60°C) |
| Temperature measurement anal | Point, Line and Area Analysis |
| Integration Protocol |  |
| Protocol type | SDK（ windows、 Linux） 、 http、onvif |
| Power supply |  |
| Typical Power Supply | DC 12v |
| Typical Power Consumption@25 °C | 1.6W |
| Connector |  |
| Video Output | Article 45 |
| Serial Communication Interface | RS485 |
| Alarm Inputs and Outputs | 1 minute 1 outlet |
| Physical characteristics |  |
| Dimensions (without lens) | 35mm*35mm*35.5mm |
| Weight (without lens) | 156.5g |
| Operating temperature | -30℃ ~ +60℃（-20°C ~60°C ensures temperature measurement accuracy） |
| Storage temperature and humidity | -45℃ ~ +75℃ ,RH≤95% |

Wuxi Bewis Sensing Technology LLC

Add： Building 30, NO. 58, Xi uxi Road, Binh u District,

Wuxi City, Jiangsu Province, China

Tel： +89 18921292620

Mail ：sales@bwsensing.com

Web：www.bwsensing.com

Network thermal imaging core

BW-TM16P series