IMU100S

Large range small volume inertial measurement units

V1.0

Technical Manual

Introduction

The IMU100S is a customised, highly accurate inertial measurement unit that measures angular velocity and acceleration parameters of moving carriers. The raw data is estimated by 6-state Kalman filtering with appropriate gain, making it suitable for inertial measurements in motion or vibration.

The IMU100S uses highly reliable MEMS accelerometers and gyroscopes with algorithms to ensure measurement accuracy, while the hermetically sealed design and rigorous production process ensure that the product can still accurately measure angular velocity, acceleration and other motion parameters of the carrier in harsh environments. The IMU100S is equipped with a digital interface, which makes it very easy to be integrated into the user's system.

Feature

- Dynamic compensation, quadrature compensation

- Sampling frequency up to 200Hz

- Baud rate 115200

Application

- Unmanned ships and underwater  robot

- Construction machinery

- Stable platform

- AGV unmanned vehicle

- RS422 output

- Wide temperature range：-40 ℃~+85℃，      Temperature compensation

- Small size：22.4×22.4×10mm

- Heavy truck

- Unmanned drive

- Robots

- Unmanned aircraft

Product Feature

Electrical index

| Power voltage | 5V DC |
| --- | --- |
| Working current | 10mA |
| Operating temperature | -40~85℃ |
| Storage temperature | -55~100℃ |

Performance index

| Gyro | Range |  | ±500°/sec |
| --- | --- | --- | --- |
|  | Full Temperature Zero Bias |  | ≤ 150 °/h |
|  | Zero Bias Stability (1σ) |  | ≤12 °/h |
|  | Zero Bias Repeatability (1σ) |  | ≤12 °/h |
| Accelerometer | Range: X,Y,Z |  | ±10 g |
|  | Zero bias at full temperature |  | ≤5mg |
|  | Zero Bias Stability (1σ) |  | ≤0.3mg |
|  | Zero bias repeatability (1σ) |  | ≤0.3mg |
|  | Supply Voltage |  | 5V |
| Systems | Operating Temperature Range |  | -40~85℃ |
|  | Shock Resistance |  | ≥2000g |
|  | Vibration |  | ≥20grms |
|  | Dimensions |  | 22.4×22.4×10mm |
|  | Weight |  | ≤30g |

Mechanical index

| Connector | Bare wire (wire length 30m) |
| --- | --- |
| Protection level | IP65 |
| Shell material | Magnesium aluminium alloy anodised |
| Installation | Three M2.5 screws |

Package product size

Product size：L22.4*W22.4*H10(mm)

Length and width may have ± 1mm error, please prevail in kind

Electrical connection

Wiring Definitions

Output Connector Wiring Definitions

| define | Description |
| --- | --- |
| VCC（+5V） | Power Positive |
| GND | Power Ground |
| RX+ | RS422 Receive Positive |
| RX- | RS422 receive negative |
| TX- | RS422 transmit negative |
| TX+ | RS422 Transmit Positive |

Order information

Product model	Communication mode	Package situation

| IMU100S |  | RS422 |  | IP65 |
| --- | --- | --- | --- | --- |

Executive standard

● National Standard (Draft) for Static Calibration Specification for Biaxial Inclination Sensors

● GB/T 191 SJ 20873-2003 General Specification for Tiltmeters and Levelling Devices

IMU100S series

Large range small volume inertial measurement units

Wuxi Bewis Sensing Technology LLC

Add：Building 30, No.58 Xiuxi Road, Binhu District, Wuxi City, Jiangsu Province, China

Tel：+86 18921292620

Email：sales@bwsensing.com Web：www.bwsensing.com